# Project Report

## BY: Singh Siddhant AkhileshSingh

## (2025PGCSIS06)

## ANISH KUMAR

## (2025PGCSIS07)

# Title: Intrusion Detection System on ESP32-VROOM2 using Optimized Artificial Neural Network

## 1. Executive Summary

This project implements an Anomaly-based Intrusion Detection System (IDS) that runs a pretrained neural network on an ESP32-WROOM-32. A PC (or phone) sends UDP packets carrying 122 numeric features to the ESP32; the device runs quantized TFLite inference and outputs a probability that the packet is an intrusion. If the model flags two consecutive packets as intrusions, an LED is lit for visibility. The whole solution demonstrates edge ML inference (TinyML) for network security use-cases.

## 2. Objectives

• Deploy a machine-learning IDS model on constrained hardware.

• Demonstrate real-time, on-device intrusion detection.

• Showcase end-to-end workflow: ML → TFLite → Microcontroller deployment.

• keep the memory and CPU usage within ESP32 limits by using an int8 quantized TFLite model embedded as a C array.

• demonstrate real-time classification of network feature vectors sent via UDP.

## 3. System Overview

Components and dataflow:

- Training environment (PC)

  - Train neural network on labelled data (normal vs attack) → produce. tflite model (quantized int8).

- Conversion

  - Convert model.tflite → C array (model.cc & model.h) for embedding into Arduino project.

- ESP32 firmware (Arduino sketch)

  - Load the model into TFLite Micro interpreter, set up WiFi AP and UDP listener, read incoming float features, normalize & quantize, run inference, print probability and trigger LED.

- Traffic sender (PC/phone)

  - Python script(s) generate and send feature vectors over UDP to ESP32 for testing & demo.

## 4. Machine Learning Model – Design & rationale

4.1 Type of IDS

This is an anomaly/behavioural IDS: the model learns patterns of normal vs attack traffic and flags deviations. It is ML-based (not signature based).

4.2 Input features

- Feature vector length: 122 floats per sample (must match model input).

- These features could be packet statistics (size, interarrival time, flags counts, protocol ports), aggregate flow metrics, or engineered features from network datasets (the exact features depend on your training dataset).

- Normalization (means & scales) must match what was used at training time. In the demo code the arrays are placeholders (means={0}, scales={1}) — in a production test they should contain real scaler values.

4.3 Model architecture (typical for this project)

- A small fully-connected neural network (MLP) is common: e.g., Input (122) → Dense (64, ReLU) → Dense (32, ReLU) → Dense (1, Sigmoid).

- Output is a single value in [0,1] interpreted as an intrusion probability.

Note: exact architecture (layers, neurons, activations) is encoded in model.cc. If you want an exact layer list, open the original training script or the. tflite metadata.

4.4 Training

- Train on labelled data consisting of normal and attack samples.

- Loss: binary cross-entropy (for probability output).

- Evaluation metrics: Accuracy, Precision, Recall, F1, ROC-AUC especially examine False Positive Rate and False Negative Rate for IDS tasks.

## 5. Model Quantization & Conversion

To deploy the trained neural network on the ESP32 microcontroller, the model must be optimized to run efficiently within strict memory and computational constraints. This is accomplished through TensorFlow Lite quantization, followed by converting the optimized model into a C-array format suitable for embedding inside the Arduino firmware. The complete process is explained below in detailed, report-ready form

## 5.1 Need for Quantization

The ESP32-WROOM-32 module has limited memory resources:

- 320 KB SRAM

- 4 MB Flash

- No floating-point unit (FPU)

- Low-power CPU (240 MHz)

Running a full-precision floating-point TensorFlow model is inefficient and often impossible due to memory limitations. To make the model deployable, the floating-point weights and activations must be converted into efficient 8-bit integer (int8) representations. This process is known as quantization.

## 5.2 TensorFlow Lite Conversion

After training the neural network in Python using TensorFlow/Keras, the model is initially saved in standard .h5 or Saved Model format. The next step is converting this model to TensorFlow Lite format using:

converter = tf.lite.TFLiteConverter.from_keras_model(model)

However, without optimization, the model remains in 32-bit floating-point form, which is incompatible with microcontroller deployment. Therefore, quantization is applied

## 5.3 Full Integer Quantization

The ESP32 requires full integer (int8) quantization, where:

- Weights are converted to int8

- Activations are converted to int8

- Inputs and outputs are also converted to int8

This reduces model size by 4× and enables the model to execute using the TFLite Micro interpreter.

Quantization compresses the model from ~1-4 MB into a much smaller int8 representation that fits into ESP32 memory and executes efficiency.

## 5.4 Exporting the TFLite Model

After conversion, TensorFlow Lite generates a compact model file:

ids_ann_int8.tflite

This file contains:

- Quantized weights

- Network structure

- Zero-points and scaling factors

- Layer metadata

These parameters are required for converting between float and int8 representations during inference.

## 5.5 Converting. tflite to C-Array (model.cc & model.h)

To embed the model inside Arduino code, it must be converted into a byte array. The TFLite Micro runtime loads this model directly from program memory.

Conversion is done using:

xxd -i ids_ann_int8.tflite > model.cc

The generated file contains:

const unsigned char ids_ann_int8_tflite[] = {

0x20, 0x00, 0x00, 0x00, ...};

const unsigned int ids_ann_int8_tflite_len = 123456;

This .cc file is added to the Arduino project, along with:

extern const unsigned char ids_ann_int8_tflite[];

extern const unsigned int ids_ann_int8_tflite_len;

in model.h

The ESP32 firmware accesses this array directly from flash memory when loading the model into the TensorFlow Lite Micro Interpreter.

## 5.6 Loading the Model on ESP32

Inside Arduino code, the ESP32 loads the model using:

const tflite: Model *model = tflite:GetModel(ids_ann_int8_tflite);

The TFLite Micro interpreter allocates tensors in a fixed 50 KB Tensor Arena, which is necessary because ESP32 does not support dynamic memory allocation for neural networks.

## 6. ESP32 Firmware Architecture

Key The firmware is designed to receive network feature vectors over Wi-Fi, process them, feed them into the embedded TFLite model, and output intrusion alerts through the Serial Monitor and the onboard LED. The system uses the ESP32 in Access Point (AP) mode, enabling direct communication without external routers.

The firmware contains the following major subsystems:

- Wi-Fi Access Point Module

- UDP Packet Listener

- Feature Preprocessing Unit (Normalization + Quantization)

- TFLite Micro Interpreter (Inference Engine)

- LED-based Detection Feedback Mechanism

- Serial Logging Module

Together, these modules enable a complete end-to-end embedded IDS pipeline.

## 6.2 Wi-Fi Access Point Module

The ESP32 is configured to operate in softAP (software access point) mode, broadcasting a local Wi-Fi network:

- SSID: ESP32_IDS

- Password: 12345678

This allows a laptop or Python script to connect directly to the ESP32. No router or Internet connection is required.

Once the AP is active, the ESP32 initializes a UDP listener on port 3333, ready to receive feature vectors.

Key code:

WiFi. SoftAP(id, password);

udp.begin(UDP_PORT);

## 6.3 UDP Packet Listener

Incoming packets from the Python script contain 122 float values (features). The UDP module:

- Waits for packets

- Checks packet size

- Reads raw bytes into a float array

- Passes the feature vector for inference

Code excerpt:

int packetSize = udp.parsePacket();

if (packetSize > 0) {

int len = udp.read((char*)features, sizeof(features));

}

If the packet does not contain exactly 122 floating-point values, the firmware ignores it.

## 6.4 Feature Preprocessing Unit

Before feeding the features into the neural network, they must be preprocessed. This includes:

## 6.4.1 Normalization

Each feature is normalized using:

normalized = (value - mean[i]) / scale[i];

Currently, placeholder values:

- means[i] = 0

- scales[i] = 1

This can be replaced with real dataset normalization values for improved accuracy.

## 6.4.2 Quantization

Since the model is int8 quantized, each normalized float must be converted to an 8-bit integer:

int8_value = round (normalized / scale) + zero point

Where scale and zero point are stored inside the TFLite model itself.

This ensures compatibility with TFLite Micro’s expected input format.
The firmware uses a helper function:

float_to_int8(norm, input tensor)

6.5 TFLite Micro Interpreter

The core inference engine is the TensorFlow Lite Micro interpreter.

6.5.1 Model Loading

The C-array model (model.cc) is loaded directly from flash memory:

const tflite::Model *model = tflite::Get Model(ids_ann_int8_tflite);

The interpreter is initialized with:

- Model pointer

- Operations resolver

- A statically allocated 50 KB tensor arena

This arena is used for:

- Model tensors

- Intermediate layer computations

- Memory required during inference

## 6.5.2 Inference Execution

Once the features are quantized and placed in the input tensor, inference is executed:

interpreter->Invoke ();

The output tensor contains a single int8 value, representing the probability of intrusion.
This is dequantized to float for interpretation:

prob = int8_to_float(output->data.int8[0], output);

## 6.6 Intrusion Detection Logic

The firmware uses a threshold-based detection mechanism:

- If probability > 0.5, the packet is classified as intrusion

- Otherwise, it is classified as normal

To avoid false positives, the firmware maintains a counter of consecutive detections:

- At least 2 consecutive intrusion predictions are required

- Once triggered, the LED stays ON for 5 seconds

This logic stabilizes detection and prevents flickering.

## 6.7 LED Alert and Serial Logging Module

LED Functionality

- LED ON → intrusion detected

- LED OFF → normal behaviour

The LED is controlled through detection timestamps and counters.

Serial Output

The Serial Monitor logs:

- Model probability

- Classification result

- Packet validity warnings

This provides real-time feedback during testing and demonstrations.

## 6.8 Execution Flow Diagram

Packet Received → Preprocess → Quantize → Inference → Probability → LED/Serial Alert

Step-by-step:

- Receive UDP packet

- Validate size (must be 122 floats)

- Normalize → Quantize

- Store input tensor

- Invoke TFLite Micro interpreter

- Read output

- Convert to real probability

- Apply detection logic

- Activate LED if intrusion

- Print results to Serial

## 8. Sending Test Traffic

Python scripts were developed to generate and send network feature vectors to the ESP32 IDS for real-time testing. These scripts use UDP communication and do not require NumPy, making them lightweight and widely compatible. Each script creates exactly 122 floating-point features, matching the input size expected by the TensorFlow Lite model on the ESP32.

Three types of traffic are simulated:

- Normal Traffic – Generates stable, low-variance feature values (around −1 to +1).
Used to test whether the IDS correctly identifies benign network behaviour.

- Attack Traffic – Produces abnormally high values (typically 5 to 15). Used to verify that the IDS detects malicious or anomalous activity.

- Mixed Traffic – Randomly alternates between normal and attack packets, with configurable delays. Ideal for demonstrations because it shows how the LED and Serial Monitor respond dynamically to changing traffic patterns.

## 9. Demonstration Steps

1. Upload the IDS sketch to the ESP32 using the Arduino IDE.

2. Open the Serial Monitor at a baud rate of 115200 to view IDS output.

3. Connect the laptop to the ESP32 Wi-Fi Access Point named ESP32_IDS.

4.  Run the Python traffic-simulation script to send normal or attack feature vectors via UDP.

5. Observe the results through the Serial Monitor logs and the ESP32’s onboard LED, which turns ON when an intrusion is detected and OFF for normal traffic.

## 10. Limitations

• Simplified Feature Input: The ESP32 does not extract real network features. Instead, feature vectors are sent externally from a Python script. This limits real-world deployment and relies on artificial testing data.

• Placeholder Normalization Values: The firmware currently uses means[i] = 0 and scales[i] = 1. Without proper feature normalization (from training data), model accuracy is reduced.

• Binary Classification Only: The system detects only two classes:

- Normal

- Attack
It cannot distinguish between different types of attacks.

• Limited Processing Power: The ESP32 cannot handle large models, complex architectures, or advanced deep learning techniques due to limited RAM and no floating-point unit.

• No On-Device Feature Extraction: The ESP32 cannot capture and parse real packets (like a full IDS such as Snort). It only performs inference, not traffic analysis.

• Only ~50 KB is allocated for inference, restricting model size and depth. The system relies on manually generated or simulated traffic through Python scripts instead of real-time sniffed network traffic. Only LED output and Serial logs are provided. No real network alerts, dashboards, or logging systems are included.

## 11. Future Enhancements

• Replace placeholder means/scales with real normalization values.

• Add multi-class attack detection.

• Implement packet sniffing + feature extraction directly on ESP32.

• Add dashboard logging or MQTT alerts.